AutoTech Motors Pvt Ltd

Vehicle Systems Requirements Specification

NGEV-2026 — Next Generation Electric Vehicle Platform

| Document Number | ATM-SRS-2026-001 |
| --- | --- |
| Version | 1.0 — Initial Release |
| Date | 25 June 2026 |
| Classification | CONFIDENTIAL |
| Project | NGEV-2026 — Next Generation Electric Vehicle |
| Client | AutoTech Motors Pvt Ltd |
| Prepared by | Systems Engineering Division |
| Approved by | Mr. Arjun Mehta — Project Director |

This document is proprietary and confidential. Unauthorised reproduction or distribution is strictly prohibited.

1. Purpose and Scope

This Vehicle Systems Requirements Specification (SRS) defines the functional, safety, and performance requirements for the NGEV-2026 Next Generation Electric Vehicle platform developed by AutoTech Motors Pvt Ltd. This document serves as the primary reference for all engineering teams, Tier-1 suppliers, and certification bodies involved in the development programme.

All requirements contained herein shall be implemented, verified, and validated before the vehicle achieves Start of Production (SOP). Requirements marked Critical are safety-relevant and subject to ISO 26262 functional safety processes.

1.1 Document Objectives

- Establish a single source of truth for all vehicle system requirements across 20+ ECUs

- Define safety integrity levels (ASIL) for all safety-relevant functions per ISO 26262

- Provide a traceable baseline linking customer needs to technical specifications

- Support regulatory homologation activities across EU, India, and selected Asia-Pacific markets

- Enable Tier-1 suppliers to develop compliant subsystem specifications

1.2 Applicable Standards

| Standard | Title | Applicability |
| --- | --- | --- |
| ISO 26262:2018 | Road Vehicles — Functional Safety | All Safety Systems |
| ISO 21434:2021 | Road Vehicles — Cybersecurity Engineering | All ECUs |
| UN ECE R100 | Electric Vehicle Safety Regulation | Battery & Drive |
| UN ECE R155 | Cybersecurity Management Systems | Connected Systems |
| ISO 14229 | Unified Diagnostic Services (UDS) | All ECUs |
| IEC 61851 | Electric Vehicle Conductive Charging System | Charging System |
| MISRA C:2012 | Guidelines for Safety-Critical C Software | All Safety SW |

2. Vehicle Programme Overview

The NGEV-2026 is a premium all-electric passenger vehicle targeting the D-segment market. The programme targets dual-motor all-wheel drive configuration, 75 kWh battery pack, 500 km WLTP range, and 0-100 km/h acceleration under 4.5 seconds.

| Domain | Key ECUs | Communication Bus |
| --- | --- | --- |
| Powertrain | VCU, MCU, BMS, PTM ECU | CAN FD @ 500 kbps |
| Chassis | Brake ECU, ESC, EPAS ECU | CAN @ 1 Mbps |
| Body | BCM, ACU, TPMS ECU | LIN @ 19.2 kbps |
| ADAS | ADAS Domain Controller, DMS ECU | 100BASE-T1 Ethernet |
| Infotainment | IVI ECU, Cluster ECU, HUD ECU | 1000BASE-T1 Ethernet |
| Thermal | TMS ECU, OBC | CAN @ 500 kbps |

3. System Requirements

The following requirements represent the complete system-level specification. Each requirement is assigned a unique identifier, system domain, priority classification, responsible ECU, and current status.

| Req ID | Domain | Requirement Description | Priority | ECU | Status |
| --- | --- | --- | --- | --- | --- |
| REQ-001 | Battery Mgmt | The system shall monitor individual cell voltages with 1mV resolution and raise fault if any cell exceeds 4.25V or drops below 2.80V. | Critical | BMS ECU | Open |
| REQ-002 | Battery Mgmt | BMS shall estimate State of Charge with ±2% accuracy and State of Health with ±5% accuracy. | High | BMS ECU | Open |
| REQ-003 | Battery Mgmt | Battery pack insulation resistance shall exceed 100 Ω/V. Isolation fault shall open main contactors within 2ms. | Critical | BMS ECU | Open |
| REQ-004 | Thermal Mgmt | Battery temperature shall be maintained between 15°C and 40°C during operation. Active cooling activates above 45°C. | Critical | TMS ECU | Open |
| REQ-005 | Thermal Mgmt | Thermal runaway shall be detected within 500ms. Emergency shutdown and driver alert shall be triggered immediately. | Critical | TMS ECU | Open |
| REQ-006 | Motor Control | Traction motor shall produce peak torque of 450 Nm and continuous torque of 250 Nm. Peak RPM shall be 18,000. | High | MCU | Open |
| REQ-007 | Motor Control | Motor controller shall support regenerative braking up to 150 kW energy recovery. FOC algorithm shall be implemented. | High | MCU | Open |
| REQ-008 | Motor Control | MCU shall respond to torque demand commands within 10ms. Anti-slip regulation (ASR) shall be implemented. | High | MCU | Open |
| REQ-009 | Vehicle Control | VCU shall process all driver inputs within 5ms. Modes: Park, Reverse, Neutral, Drive, Sport. | Critical | VCU | Open |
| REQ-010 | Vehicle Control | VCU shall comply with ISO 26262 ASIL-D functional safety requirements. Watchdog timer timeout shall be 10ms. | Critical | VCU | Open |
| REQ-011 | Vehicle Control | VCU shall support OTA software updates without interrupting vehicle operation. | Medium | VCU | Open |
| REQ-012 | ADAS | Forward collision warning (FCW) shall activate at speeds above 30 km/h. AEB shall stop vehicle from 50 km/h within 15 metres. | Critical | ADAS ECU | Open |
| REQ-013 | ADAS | Lane departure warning (LDW) shall detect lane markings with 95% accuracy. Lane centering shall maintain ±30cm from centre. | High | ADAS ECU | Open |
| REQ-014 | ADAS | Driver monitoring system (DMS) shall detect drowsiness within 5 seconds and alert driver via audio and visual warning. | High | ADAS ECU | Open |
| REQ-015 | Charging | On-board charger shall support AC charging at 11 kW and 22 kW (single and three phase) with minimum 94% efficiency. | High | OBC | Open |
| REQ-016 | Charging | Vehicle shall support DC fast charging via CCS2 connector. Battery shall be pre-conditioned before fast charging. | High | OBC | Open |
| REQ-017 | Charging | Charging system shall support vehicle-to-grid (V2G) bidirectional charging and scheduled charging via mobile app. | Medium | OBC | Open |
| REQ-018 | Diagnostics | All ECUs shall support UDS (ISO 14229) diagnostic protocol. OBD system shall comply with OBD-II standard (SAE J1979). | High | All ECUs | Open |
| REQ-019 | Diagnostics | System shall support predictive maintenance alerts based on live sensor data. Remote diagnostics via telematics unit. | High | VCU / Cloud | Open |
| REQ-020 | Cybersecurity | Vehicle shall comply with ISO 21434 and UNECE WP.29 cybersecurity regulations. All ECU firmware shall be cryptographically signed. | Critical | Gateway ECU | Open |
| REQ-021 | Cybersecurity | All external communication interfaces shall use TLS 1.3 encryption. Intrusion detection system (IDS) shall be implemented. | Critical | Gateway ECU | Open |
| REQ-022 | Powertrain | Vehicle shall achieve 0-100 km/h in under 4.5 seconds. Top speed limited to 250 km/h. WLTP range minimum 500 km. | High | PTM ECU | Open |
| REQ-023 | Powertrain | Powertrain shall support one-pedal driving with full regenerative deceleration. Eco, Normal, and Sport drive modes required. | Medium | PTM ECU | Open |
| REQ-024 | Connectivity | Vehicle shall have embedded 4G LTE modem. Infotainment shall support Wi-Fi 6, Bluetooth 5.0, Apple CarPlay, and Android Auto. | Medium | IVI ECU | Open |
| REQ-025 | Safety | Vehicle shall achieve 5-star Euro NCAP rating. Minimum 6 airbags standard. eCall functionality per EU 2015/758 mandatory. | Critical | ACU / VCU | Open |

4. Programme Milestones

The following milestone schedule represents the agreed programme plan referenced from programme kickoff date. All milestones shall be formally reviewed in the Programme Review Board (PRB) meeting.

| Milestone | Target | Exit Criteria |
| --- | --- | --- |
| M1 — System Architecture Review | Week 4 | Architecture sign-off, HARA complete, ASIL ratings assigned |
| M2 — Software Requirements Baseline | Week 8 | All software requirements reviewed and approved by client |
| M3 — Hardware Design Freeze | Week 12 | PCB schematics, BOM, and mechanical drawings finalised |
| M4 — Software Integration Build 1 | Week 18 | First integrated software build on target hardware |
| M5 — HIL Testing Complete | Week 24 | All ECU functions validated on Hardware-in-the-Loop rig |
| M6 — Vehicle Integration | Week 30 | Full vehicle build complete, first drive achieved |
| M7 — System Validation Testing | Week 36 | All safety goals validated, ISO 26262 confirmation measures complete |
| M8 — Homologation Submission | Week 42 | Regulatory submissions complete — Euro NCAP, WLTP, UN ECE |
| M9 — SOP Readiness Review | Week 48 | Production readiness confirmed, all open issues closed |
| M10 — Start of Production (SOP) | Week 52 | First customer vehicle produced |

5. Document Approval

This document requires formal approval from all signatories listed below before it is considered the authorised baseline. Subsequent changes shall follow the Change Request (CR) process and require re-approval.

| Name | Role | Signature | Date |
| --- | --- | --- | --- |
| Mr. Arjun Mehta | Project Director |  |  |
| Ms. Priya Sundaram | Systems Engineering Lead |  |  |
| Mr. David Krishnan | Software Architecture |  |  |
| Ms. Ananya Rajan | Safety & Homologation |  |  |

— END OF DOCUMENT —

AutoTech Motors Pvt Ltd  |  ATM-SRS-2026-001  |  Version 1.0  |  © 2026 AutoTech Motors Pvt Ltd